Task: Bridge & Tunnel Infrastructure Density Map — Karnataka

We need a spatial analysis that maps the concentration of engineered road infrastructure, bridges, and tunnels across Karnataka, and assesses which National Highway corridors carry the heaviest maintenance burden. You will work with three layers loaded in the project: Karnataka.gpkg (state boundary), karnataka_major_roads.gpkg (42,475 OSM line features with bridge and tunnel fields), and national_highways.gpkg (3,119 features with ref and length_km). Reproject all three to EPSG:32643 (WGS 84 / UTM Zone 43N), just as the project CRS.

Start by extracting bridge and tunnel segments separately from karnataka_major_roads using expression filters (bridge = 'T' and tunnel = 'T'). Add a field SEG_LEN_M to each extracted layer, computed as $length, this drives all downstream density and burden calculations. Save these as bridge_segments.gpkg (blue line, width 0.8) and tunnel_segments.gpkg (purple line, width 0.8).

A 20 km hexagonal grid must be generated over Karnataka using Create grid, with grid type set to Hexagon, horizontal and vertical spacing both set to 20,000 metres, and the extent set to match the Karnataka.gpkg layer in EPSG:32643 (WGS 84 / UTM Zone 43N). it must be clipped to Karnataka.gpkg, using Clip, so boundary hexagons are trimmed to the Karnataka boundary. Spatial summary join must also be run using join by location summary with an intersects predicate to sum SEG_LEN_M from bridge segments onto each cell, then repeat for tunnel segments. From these sums compute per cell: BRIDGE_KM and TUNNEL_KM (each sum divided by 1,000), TOTAL_INF_KM (their sum), and TOTAL_DENS as TOTAL_INF_KM divided by cell area in km². Classify each cell into INFRA_CLS using these data-calibrated thresholds: Very High (≥ 0.006), High (≥0.002<0.006), Moderate (≥0.0005<0.002), Low (> 0), None (= 0). Save as infra_density_hex.gpkg with a YlOrRd graduated renderer on TOTAL_DENS.

For the National Highways (NH) burden layer, dissolve national_highways.gpkg by ref to reconstruct 83 corridor lines and compute NH_KM from geometry. Use the same summary join approach to intersect bridge and tunnel segment lengths onto each dissolved corridor, producing BRIDGE_KM, TUNNEL_KM, and INFRA_KM (their combined total). Compute INFRA_PCT as (INFRA_KM / NH_KM) × 100 — the share of each corridor's length occupied by engineered structures. Classify into BURDEN_CLS: Critical (≥ 20%), High (≥10<19.9%), Moderate (≥5<9.9%), Low (< 5%). Save as nh_infra_burden.gpkg with a categorized line renderer on BURDEN_CLS using the PuBu colour ramp.

The project must deliver a project folder, Bridge_Tunnel_Infrastructure_Density/ containing the input layers (Karnataka.gpkg, national_highways.gpkg, and karnataka_major_roads.gpkg), the intermediate layers (bridge_segments.gpkg and  tunnel_segments.gpkg), the output layers (infra_density_hex.gpkg and nh_infra_burden.gpkg ), and a QGIS project, Bridge_Tunnel_Infrastructure_Density.qgz, saved in a relative path, with all the expressions and symbology preserved, and must open without broken links. The reference_layout.png only provides a visual guide and has no analytical relevance in the project.